Manual for N_eye (iPhone)   

Contents

3

1. Introduction

3

1.1 Function introduction

3

1.2 Runtime environment

3

2. Installation

4

3. Operation

4

3.1 Home interface

5

3.2 Device management

5

3.2.1 Device list

6

3.2.2 Add device

8

3.2.3 Edit/Delete Device

9

3.3 Live view

11

3.3.1 PTZ control

12

3.3.2 Panel button description

13

3.4 Snapshot and Record files

15

3.5 Remote Record file Search and playback

17

3.6 Setting



1. Introduction

1.1 Function introduction

N_eye is your best mobile surveillance system for iPhone. With your sensitive finger tips you can control camera zoom or PTZ, 4 Channel live camera videos help you grasp important scenes with ease. N_eye is convenient features unleash the power of your DVRs and IP cameras. It has the following features include:
1)  Support both Wi-Fi, 3G and 4G wireless network connection

2)  Device management

3)  Support QR Code scanning and LAN searching to add device

4)   Live video support 1, 4 channels display on one screen

5)  Full screen play and support both landscape and portrait mode

6)  PTZ using finger touch, image multiplication using two fingers

7)  Snapshot and playback;
8)  Video record and playback 
9)  Support two-way voice

10)  Support remote video playback

11)  Support remote alarm functions

1.2 Runtime environment

Operating system: ios

Version: ios 5.0 or above 

Applicable equipment: iPhone 

The network environment: Wi-Fi, 3G, 4G 

2. Installation

Search Keyword “N_eye” in your “App Store”, then you will get the installation package of this application. Tap “[image: image1.png]


”to install. 

After the installation is complete, the name of “N_eye” icon will show on your iPhone’s desktop, as below:

[image: image2.png]


     [image: image3.jpg]------




P1                                    P2

3. Operation

3.1 Home interface
Tap N_eye’s icon to run the program, after the initial screen, you will be brought to the home interface, as below:

[image: image4.png]


   

P3 home interface

3.2 Device management
3.2.1 Device list

Tap “[image: image5.png]


” button to spread out all the function buttons, then tap “[image: image6.png]


” button, the device list view will appear.

[image: image7.png]Device

A
worisens ®

B

N ®




P4

3.2.2 Add device

Tap “[image: image8.png]


” button in the home interface view or “[image: image9.png]


” button in the device list view, to open the device adding view.

[image: image10.jpg]QR Code Scan

Manual

Local Search

Cancel





P5

Here provides 3 methods to add devices: 

	Button
	Descriptions

	QR Code Scan
	Scan the OR code on your device to fill in the GID string,

	Manual
	Fill in the info manually

	Local Search
	If the device is on the same local network with your phone, GooLink would find them and let you add those devices easily


[image: image11.png]Name: testB.
GID: 2/01602d07
User: admin

Channel Number

O Il e e[ [omo





P6 Manual way

	Entry
	Descriptions

	Name
	Any characters that you like to identify this device. Please be noted

that the name should not duplicate with existing device’s name.

	GID
	The device GID (if you are using the QR Code Scan/ Local Search method to add device, this item will be fill automatically)

	User
	The device’s login name

	Password
	The device’s login password

	Channel Number
	Select the channel number, when assign “auto”, the app will set the number automatically.


When you finish adding devices, they will show in the home interface view under the playing windows area. If the device icon’s color is bright-Blue, it means the device is online.

[image: image12.png]@ﬁucucuc

teStA  testB  testC  testD

o000
+ @ 8 QD & -





P7

3.2.3 Edit/Delete Device
     Tap “[image: image13.png]


” button in the Device list (see P4) to Edit/Delete a device, and you will see the page like Picture-8 as below. Here you can change the information and tap “Done” to save, or tap “Cancel” to return to the device list without changing.

     If you want to delete the device, just tap the red button “[image: image14.png]Delete



”.

[image: image15.png]Name: testB
GID: 201602007
User: admin

Channel Number





P8

3.3 Live view

Step one: Tap the device icon which you want，N_eye will check out the channels of this device, and show the channel icons under the device area. 

Step two: Tap the channel icon which you want, then the channel’s video will be connected.

Repeat above steps to connect to other channels of the device or another device.
When the channel selected, the channel circle icon’s color will turn into bright-Blue like [image: image16.png]


. Tap the bright-Blue icon, the channel video will stop and the channel circle icon color becomes grey.

[image: image17.png]@ﬁucucuc

teStA  testB  testC  testD

o000
+ @ 8 QD & -




     [image: image18.png]testD-CH1

@---(

testA testB testC

$ @ 48 QD & .




P9                                    P10

Tips: You can tap and drag the window to swap any two displaying videos.
P11 landscape mode
3.3.1 PTZ control

1）finger-touching over the displaying video

[image: image19.png]Y e e




P12 direction rotation
[image: image20.png]



P13 image multiplication
2）Tapping the PTZ control buttons
Tap [image: image21.png]


 to open the PTZ Control panel. Slide to show more PTZ control button; To hide the panel, you can tap the button again.
[image: image22.png]testD-CH1

@---(

@ E ol Ed o el
& B a D & .




P14 PTZ Control panel
3.3.2 Panel button description
	The button icon
	Function keys

	[image: image23.png]



	Add a new device

	[image: image24.png]



	[image: image25.png]



	PTZ:UP

	
	[image: image26.png]



	PTZ:Down

	
	[image: image27.png]



	PTZ:Right

	
	[image: image28.png]



	PTZ:Left

	
	[image: image29.png]



	Zoom In

	
	[image: image30.png]



	Zoom Out

	
	[image: image31.png]



	Focus Add

	
	[image: image32.png]r
L)




	Focus Minus

	
	[image: image33.png]



	Iris Add

	
	[image: image34.png]



	Iris Minus

	[image: image35.png]



	Snapshot

	[image: image36.png]



	Start/Stop Record

	[image: image37.png]



	Audio

	[image: image38.png]



	Talkback

	[image: image39.png]



	[image: image40.png]



	Device List

	
	[image: image41.png][}




	Snapshot and Record files

	
	[image: image42.png]



	Remote Record file Search and playback

	
	[image: image43.png]



	Local Settings

	
	[image: image44.png]



	User gGIDe


3.4 Snapshot and Record files
Tap “[image: image45.png][}



” to visit the Snapshots & Records view, you can playback or delete the files from this view.
[image: image46.jpg]Lt

LI

testA

testC

testD

testE



     [image: image47.jpg]2014_02_13_17_18_25.mp4
testA, channel: 01, size: 44Kb

2014_02_13_17_19_02.mp4
testA, channel: 01, size: 334Kb

2014_02_13_17_19_22.mp4
testA, channel: 01, size: 399Kb

2014_02_13_17_19_45.mp4
testA, channel: 01, size: 22Kb

2014_02_13_17_19_46.mp4
testA, channel: 01, size: 4Kb

2014_02_13_17_19_47.mp4
testA, channel: 01, size: 23Kb

2014_02_13_17_19_48.mp4
testA, channel: 01, size: 4Kb





P15                                    P16

 [image: image48.jpg]2014_02_13_17_18_22.png
testA, channel: 01, size: 358Kb

2014_02_13_17_18_23.png
testA, channel: 01, size: 389Kb

2014_02_13_17_18_58.png
testA, channel: 01, size: 344Kb



     [image: image49.jpg]14_02_13_17_18_22.png
tA, channel: 01, size: 358Kb

2014_02_13_17_18_23.png
testA, channel: 01, size: 389Kb

2014_02_13_17_18_58.png
testA, channel: 01, size: 344Kb




P17 detele file

3.5 Remote Record file Search and playback
Tap "[image: image50.png]


" , then click any device on “Device” page to open the device’s “Remote Record Search” view. Before performing a search, user can customize the search conditions like "Channels, StartTime, EndTime, VideoType", as below:

[image: image51.png]testA
sorersces ®

testB

01602407

testC
soter ®

€]



     [image: image52.jpg]Remote Record Search

Channels All
StartTime 2014-06-11-00-00-00
EndTime 2014-06-11-23-59-59

Switch alarm Record
Motion detection recording
Conventional Record

Manual recording

L S SR SRR S

All Record




P18
[image: image53.jpg]/idea0/2014-06-11/001/12.00.50-12.00.55[R][@40][0].h264

/idea0/2014-06-11/001/12.00.56-12.01.29[R][@44][0].h264

/idea0/2014-06-11/001/12.01.29-12.05.00[H][@51][0].h264

/idea0/2014-06-11/001/12.05.00-12.10.00[H][@9c][1].h264

/idea0/2014-06-11/001/12.10.00-12.15.00[H][@102][0].h264

/idea0/2014-06-11/001/12.15.00-12.20.00[H][@16b][1].h264

/idea0/2014-06-11/001/12.20.00-12.25.00[H][@1d1][0].h264

/idea0/2014-06-11/001/12.25.00-12.30.00[H][@239][0].h264

/idea0/2014-06-11/001/12.30.00-12.35.00[H][@2a2][1].h264




     [image: image54.png]C

< Back

11 12:00:54





P19
3.6 Setting

Tap “[image: image55.png]


” to open the “System Settings” view:

[image: image56.png]| System Settings Done

Video Views
Live Preference

View Preference




P20

	Video Views
	[image: image57.png]



	One window display in the Live view

	
	[image: image58.png]



	Four windows display in the Live view

	Live Preference
	[image: image59.png]ZEEIRGEY  Fluent I





	No buffering, which means there is minimum delay of the video display and video fluency may be suffered;

	
	[image: image60.png]



	Buffering enabled, the video will be more fluent in this mode;

	Mode Preference
	[image: image61.png][ original R ]





	Watching video and saw the size of the picture is directly by the devices to get the size of the screen,

	
	[image: image62.png]



	Watching video and saw the size of the picture is from the equipment drawing show the effect of the picture,


Step one





Step two









6

