
Keyboard Quick Start Guide
                                                     Version 1.0.0
Table of Contents

11
Features and Specifications


11.1
Features


11.2
Front Panel


31.3
Rear Panel


31.3.1
NKB1000 Series


31.3.2
KB1000 Series


31.3.3
KBD1000 Series


41.4
Port Features


41.5
Power


52
Menu Operation


52.1
Boot up and Shut down


52.1.1
Boot up


52.1.2
Shut Down


52.2
Main Menu List


62.3
Menu Operation


62.3.1
System


82.3.2
Control point


92.3.3
Account


112.3.4
Config


112.3.5
Version


112.3.6
Model


122.4
Zones


133
Keyboard Control DVR


133.1
Connection


133.1.1
RS232 Serial Port Connection


133.1.2
Via RS485 Connection


133.1.3
Multi-keyboard link via RS485


143.2
Network Connection (For network keyboard only)


143.2.1
RJ45 port connection


153.3
DVR and Keyboard Setup


153.3.1
DVR Setup


153.3.2
Network Connection Setup


163.4
Operation


163.4.1
Login


163.4.2
Logout


163.4.3
Playback


163.4.4
Record


163.4.5
PTZ Control and Video Color


173.4.6
Tour


173.4.7
1-Window/Multiple-Window Switch


184
Network Keyboard Control Speed Dome


184.1
Cable Connection


184.2
Keyboard Setup


184.3
Operation


184.3.1
Speed Dome Menu


194.3.2
Iris/Zoom/Focus


194.3.3
Preset


194.3.4
Scan


204.3.5
Pan


204.3.6
Tour


214.3.7
Pattern


225
Keyboard Control NVS


225.1
Connection


225.2
Keyboard Setup


225.3
Operation


225.3.1
Login


225.3.2
Exit


225.3.3
1-Window/Multiple-Window Switch


246
Keyboard Upgrade


246.1
Control Keyboard Upgrade


246.2
Network Keyboard Upgrade


257
Make COM Cable


257.1
Make RS232 COM Cable


257.2
Make RS485 COM Cable


257.3
25-pin Port and Switch Box Connection




Welcome

· Thank you for purchasing our keyboard!

· This quick start guide is designed to be a reference tool for the operation of your system. 

· Before the operation please read the following safeguards and warnings carefully!

· Please keep it well for future reference.
Important Safeguarding and Warnings 

1．Electrical safety

All installation and operation here should conform to your local electrical safety codes. 

We assume no liability or responsibility for all the fires or electrical shock caused by improper handling or installation. 

2．Transportation security

Heavy stress, violent vibration or water splash are not allowed during transportation, storage and installation.

3．Installation

Keep upwards. Handle with care. 

Do not place objects on the network keyboard

4．Qualified engineers needed


All the examination and repair work should be done by the qualified service engineers.  

We are not liable for any problems caused by unauthorized modifications or attempted repair.

5．Environment

The keyboard should be used in a cool, dry place away from direct sunlight, inflammable, explosive substances and etc.

6.  Accessories

Be sure to use all the accessories recommended by manufacturer.
Before installation, please open the package and check all the components are included.

1 Features and Specifications
1.1 Features
This series keyboard has the following features 
· One keyboard can control multiple DVRs, or you can use multiple keyboards to control one DVR.

· Support dome operation.
· Support RS485 port and RS232 port.
· Support PTZ operation via joystick

· Support all operations you get from DVR front panel function keys

· Support keyboard lock function 
· Support multiple-level operation  rights
· Support cascade connection
· Support single or multiple DVRs’s menu setup
· On-screen menu, user-friendly procedure. It is very easy for you to operate
Besides above mentioned features, network keyboard also has the following features: 
· Can control network video server  and network video decoder

· Support network ports connection
1.2 Front Panel
The keyboard front panel is shown as below. See Figure 1‑1.

[image: image1.png]



Figure 1‑1
Please refer to the following sheet for detailed information. 
	SN
	Note
	Icon
	Button Name
	Function 

	1
	General function panel 
	ESC
	ESC
	Cancel current operation

	
	
	
	
	In playback mode, click this key to go back to dynamic surveillance mode.

	
	
	SETUP
	Switch mode 
	It is to switch between: SETUP/SETUP SET mode. 
In SETUP mode, system supports 【MULT】, 【CAM】,【PTZ】 combination operation. 

	
	
	SHIFT
	Change input method 
	Switch between input methods

	
	
	ENTER
	Confirm button 
	Confirm current operation

	
	
	0～9
	Character and numerals
	Switch between numeral/characters, click 

Shift to switch.

	2
	DVR control panel 
	ID
	ID shortcut 
	Shortcut key to go to ID zone search

	
	
	CAM
	Device channel number
	Control DVR channel.

	
	
	MULT
	Split button 
	Control DVR window split

	
	
	AUX
	Auxiliary 
	Go to video color interface and open DVR local PTZ interface. 

	
	
	FN
	Function key 
	Work with the number buttons. 
【FN】+1: Go to DVR assistant interface. 
【FN】+2: Tour 

	
	
	PTZ
	PTZ button 
	Control channel PTZ.

	
	
	PLAY
	Playback button 
	Control DVR　playback

	3
	Speed dome control panel 
	IRIS
	iris
	Control iris.

	
	
	FOCUS
	focus
	Control focus 

	
	
	ZOOM
	zoom
	Control zoom. 

	
	
	OTHER
	Speed dome menu 
	Go to speed dome menu.

	
	
	PERSET
	Preset
	Go to preset setup interface.

	
	
	SCAN
	Scan
	Go to scan setup interface.

	
	
	PAN
	Pan
	Go to pan setup menu. 

	
	
	TOUR
	Tour
	Go to tour setup menu. 

	
	
	PATTERN
	Pattern 
	Go to pattern setup menu. 

	4
	Joystick

	-
	-
	Aux menu and function operation. 

	5
	Indicator light panel
	PWR
	Power indicator light 
	The light is on when the keyboard is properly connected to the power supplying. 

	
	
	TR/TX
	Network data receive and send indicator light 
	The light is flashing when the keyboard is connected to the network. 

	
	
	NET
	Network indicator light 
	The light is on when the device is connected to the network. 

	
	
	232
	RS232 indicator light 
	The light is flashing when there is 232 data transmission. 

	
	
	485
	RS485 indicator light
	The light is flashing when there is 485 data transmission.

	
	
	USB
	USB indicator light 
	The light is on when the keyboard is connected to the USB device. 
· For NKB1000 series product, the light is on when the keyboard is connected to the PC. 
· For KB1000 and KBD1000 series product, the light is on when the keyboard is connected to the flash disk. 

	6
	LCD
	-
	-
	OSD menu


1.3 Rear Panel
1.3.1 NKB1000 Series 

Please refer to Figure 1‑2  for keyboard port information. 

[image: image2.png]Eﬂﬂﬂﬂlﬂﬂﬂ





Figure 1‑2
Please refer to the following sheet for detailed information. 

	SN
	Name

	1
	USB port 

	2
	RS422 port

	3
	RS485 port

	4
	Network port

	5
	RS232 port 

	6
	Power socket 


1.3.2 KB1000 Series 

Please refer to Figure 1‑3  for keyboard port information. 

[image: image3.png]2 3

R+ R- T- T+[A1B1 A2 B2

U

1
=L
<

H e





Figure 1‑3
Please refer to the following sheet for detailed information. 

	SN
	Name

	1
	USB port 

	2
	RS422 port

	3
	RS485 port

	4
	Power socket


1.3.3 KBD1000 Series 

Please refer to Figure 1‑4  for keyboard port information. 

[image: image4.png]2 3

R+ R- T- T+[A1 B1 A2 B2

1
) ([T
g





Figure 1‑4
Please refer to the following sheet for detailed information. 

	SN
	Name

	1
	USB port 

	2
	RS422 port

	3
	RS485 port

	4
	RS232 port 

	5
	Power socket


1.4 Port Features
· RS232 port can directly connect to device if there is RS232 port. The distance should be within 10m. 
· RS485 transmission distance is 1200 m（9600bps）, max 3000m actually. 
· For network keyboard series, please connect to the network port to connect the DVR or the network speed dome. 
1.5 Power

The keyboard uses +12V DC, you can use +12V adapter included in the accessories bag. 
2   Menu Operation 
2.1 Boot up and Shut down
2.1.1 Boot up 

Connect the keyboard to the proper power supplying source. The power indicator light is on. You can see the welcome interface after the device properly booted up. 
Click 【ENTER】 and 【SETUP】 button to go to the following interface. See Figure 2‑1.
[image: image5.png]adnin





Figure 2‑1
Input user name and password, system goes to the main menu. 

Note
Factory default user names are: admin/default/guest. The password is: 888888.
The default can not add/delete user, modify user rights or manage rights. 

The guest can only operate control points. There are no other rights. 

2.1.2 Shut Down

After you exit the system, unplug the power cable to turn off the device. 

2.2 Main Menu List

The main menu list is shown as below. See Figure 2‑2.
[image: image6.jpg]E]

Time

Netwark
System
r M| Address
Menu Settings
A
System Upgrade
Loga Upgrade
Zone

Modify Pwe

Add User

Accounte

Del User

Modify Rights

Importe

Config

Exporte

Clear Zanes

Version

Madel

D

Name

IP Address

—| Zone Contral

232 Address

485 Address

Default





Figure 2‑2
2.3 Menu Operation 
Use the joystick on keyboard to move up/down/left/right to select items. 

Use keyboard to input number or character to set parameters. 

Click 【ENTER】 to confirm current operation. Click【ESC】 to exit. 

2.3.1 System 
The system manager includes time setup, network setup, address setup, aux setup, system update and logo update. See Figure 2‑3.
[image: image7.png]@Network ®Logolpdate
@address
@iux





Figure 2‑3
2.3.1.1 Time 

It is to set keyboard system time. It includes year, month, date, minute and second. See Figure 2‑4.
[image: image8.png]mo=m

2014 w
11
16
10





Figure 2‑4
2.3.1.2 Network 

It is to set keyboard IP address, subnet mask, gateway, port(Default setup is 37777) and etc. Please set according to your actual situation. See Figure 2‑5.
[image: image9.png]1P Addr: Gatevay:

e 2. 16 9. 0. 1
Subnet Hask Port: 87777
255.255. 0. 0





Figure 2‑5
2.3.1.3 Address 
The address value ranges from 0 to 255. The default setup is 0. When there is cascade connection, the 0 has the highest priority and the 255 has the lowest priority. See Figure 2‑6.
[image: image10.png]



Figure 2‑6
2.3.1.4 Aux 
It is to set screen backlight and auto logout function. See Figure 2‑7.
· Backlight: It is to set screen backlight. The options are: on/off.

· Auto logout: After you enable the auto logout function, system can auto logout when there is no operation for specified time. 

[image: image11.png]



Figure 2‑7
2.3.1.5 System Update 
[image: image12.png]



Do not remove the flash disk during the whole process. Otherwise it may result in upgrade failure and the network keyboard can not boot up properly.
The flash disk upgrade function is for KB1000 or KBD1000 series product. 

Please select system update interface and click 【ENTER】 button to go to the update interface. Follow the prompt to click the 【ENTER】 to update. 

2.3.1.6 Logo update

The logo upgrade function is for KB1000 or KBD1000 series product. You can update logo file if necessary. 

2.3.2 Control point 

It is to set control point information. See Figure 2‑8.  All the items listed below shall be set. You can use the ID, device name, COM address or the IP address to search a device easily in chapter 2.4.
[image: image13.png]



Figure 2‑8
[image: image14.png]



The information you input here shall be unique. Otherwise, system can not save the setup.
· ID: It is device ID number so that you can recognize device quickly. 
· Name: Customized device name. 

· Type: Device type. The options include: SD/NVS/DVR.
· Link: The options include: NET/RS232/RS485. Please select the type and set the corresponding parameters. The interface is shown as in Figure 2‑9.
[image: image15.png]Type

NET

Zone IP
Port
Protocol
Password

RS232 RS485
RS232 Address RS485 Address
Baud Rate Baud Rate
Protocol Protocol
Data Bit Data Bit
Parity Pairty
Stop Bit Stop Bit




Figure 2‑9
[image: image16.png]



PLEASE MAKE SURE THE COM TYPE AND THEN PARAMETERS YOU SET HERE IS THE SAME AS THE ACTUAL SITUATIONS. OTHERWISE, THE KEYBOARD CAN NOT CONTROL THE DEVICE!
When your link mode is net, please input corresponding administrator password of the SD/DVR/NVS to login the controlled device. The default user name is admin. See Figure 2‑10.

[image: image17.png]-

adnin





Figure 2‑10
2.3.3 Account 

Here is for you to change password, add user, and delete user and its rights. See Figure 2‑11.
[image: image18.png]@add User
@Del User
@Modify Rights





Figure 2‑11
2.3.3.1 Change password 

Select a user and then input new password. Usually we recommend 6-digit password and you can change the input method to input password value. Input password again to confirm. You can see the corresponding dialogue box if your two passwords input here are not the same. Please click 【ESC】 to return to modify and click 【ENTER】 button to save. See Figure 2‑12.
[image: image19.png]adnin





Figure 2‑12
2.3.3.2 Add Users
Please input user name (max 8-digit) and then set corresponding password. The group options include user/guest. See Figure 2‑13,
[image: image20.png]User:





Figure 2‑13
· Guest: The guest can only operate control point operation. 

· User: The user has the PTZ control, system setup, backup, advanced setup and etc. Move the cursor to the corresponding item and then use the joystick to move the left/right to turn ( to ( to enable this function. See Figure 2‑14.
[image: image21.png]®FTZ Control

OBackup
O Advanced





Figure 2‑14
Note
The account setup here refers to the rights for the keyboard to control device. The keyboard can not control the device to operate specified function if it does not have the corresponding rights. 
2.3.3.3 Delete Users 

Select a user first and then click 【ENTER】 to remove. 
2.3.3.4 Rights

It is to change user rights. 
2.3.4 Config 
Note

The configuration files import/export function is for KB1000 and KBD1000 series product only. 
The configuration manager includes configuration files import/export, clear zones and factory default setup. See Figure 2‑15.
[image: image22.png]Export
@Clear Zones
@Default





Figure 2‑15
2.3.4.1 Import

Insert the flash disk that contains the upgrade files to the keyboard and the go to the import interface. Click 【ENTER】 to import the configuration files. 

2.3.4.2 Export

Insert the flash disk to the keyboard and then go to the export interface. Click 【ENTER】 to export the configuration files. 

2.3.4.3 Clear Zones
Go to the clear zones interface and then click 【ENTER】 to clear all control points. 

2.3.4.4 Default

Go to the default interface and then click 【ENTER】 to restore keyboard configuration to factory default setup. 

2.3.5 Version 
It is for you to view current version information and etc. See Figure 2‑16.
[image: image23.png]Version:

2. 606, 0024. 0
Built Date:
2013-11-16





Figure 2‑16
2.3.6 Model

It is for you to view current keyboard model. See Figure 2‑17.
[image: image24.png]Model :





Figure 2‑17
2.4 Zones
In this interface, you can quickly search a device via device ID, device name, COM address, IP and etc. See Figure 2‑18.
[image: image25.png]3. IPAddr

D
Nane
4. 232hddr





Figure 2‑18
3 Keyboard Control DVR
3.1 Connection 

3.1.1 RS232 Serial Port Connection 

You can use one RS232 cable to connect DVR serial port and keyboard serial port together.  
DVR address number ranges from 1 to 255，please refer to DVR user’s manual for more information.
3.1.2 Via RS485 Connection 
You can use one RS232 port connects to one DVR only. 
You need to use RS 485 pot if you want to control several DVRs.
Connection 
You can user one RS485 to RS232 switch box (provided) connect with DVR RS232 port. See Figure 3‑1.

[image: image26.png]RS485-RS232 Switch
Box

RS485-RS232 Switch
Box

RS485-RS232 Switch
Box





Figure 3‑1
Note

One RS485 bus can connect with thirty-two RS485-RS232 switch boxes，one RS485-RS232 switch box can connect with twelve DVRs. 
3.1.3 Multi-keyboard link via RS485
You can use the following figure if you are connecting multiple keyboards via RS485 port. See Figure 3‑2.
[image: image27.png]Other RS485
devices





Figure 3‑2
Multi-converter connection：
The last converter on RS 485 port must be short circuited. (E.g.: open converter). See Figure 3‑3.
See the image below, you need to change [1-2] short circuit to [2-3] short circuit.

Note:

Menu serial setup should conform to the device you connected; otherwise keyboard can not control the device!
[image: image45.png]



Figure 3‑3
3.2 Network Connection (For network keyboard only)
Connect RJ45 port to the network to realize network control and upgrade.

3.2.1 RJ45 port connection
3.2.1.1 Direct connection
Please refer to Figure 3‑4 for direct connection information. 

[image: image28.png]Network Cable
(Crossover)




Figure 3‑4
3.2.1.2 Connect via HUB
Please refer to the following figure if you are connecting via the hub. See Figure 3‑5.[image: image29.png]DVR

Network cable
(Straight Through )

DVR




Figure 3‑5
3.3 DVR and Keyboard Setup 
3.3.1 DVR Setup 
a) Before operation, please make sure cable connection is proper.

b) In DVR menu, from Setting->RS232. The COM function is network keyboard and then set corresponding parameters. 
The default setup is shown as below.
· Baud rate: 9600

· Data bit: 8

· Stop bit: 1

· Parity: none.

Please note keyboard setup should conform to DVR setup.

c) Go to the zones interface of the keyboard. Please refer to chapter 2.3.2
Note

· RS232 address/RS485 address is the device No. of the DVR.

· The protocol usually is DH-2. It is the network keyboard and the 2nd control keyboard protocol.  The DVR COM protocol is network keyboard protocol.
d) After the setup, click 【ESC】 to exit control point control interface. System pops up a dialogue box you can click 【ENTER】 to save data or click 【ESC】 to cancel setup. 
3.3.2 Network Connection Setup 
The network connection steps are similar to the COM setup. Please note, on the zones interface, the link type shall be NET.

Please input DVR IP address, the control point port is 37777. The protocol is the same as the COM setup. 
3.4 Operation 
3.4.1 Login 
[image: image30.png]



The DVR local user has the higher priority than then keyboard user. Before you login, please logout the DVR local menu user, otherwise, you can not use the keyboard to control the device.

You can select ID, device name, IP address, RS232, RS485 and etc to login. 

a) Click 【ENTER】 on the keyboard, you can go to the menu options interface.

b) Select zones.

c) Input ID, device name, IP address.RS232, RS485 you set on control point interface,  Click 【ENTER】 to begin search.
· System can automatically connect to the DVR if the search is successfully. Please note you need to input corresponding administrator password of the controlled device to login. See Figure 2‑10. 
· System pops up a dialogue box to remind you such as the address does not exist, please input again. Please check the information and input again. 
Tips

Click 【ID】 on the keyboard and then input ID number. Click 【ENTER】 to quickly find the device. 

3.4.2 Logout 

Click 【ID】 on the keyboard and then click 【ESC】 to exit. 
3.4.3 Playback 

Click 【PLAY】 on the keyboard, you can go to the DVR search interface. Use joystick to control the playback buttons such as stop, fast play, slow play and etc. Click 【ENTER】 to confirm. See Figure 3‑6.
[image: image31.png]94

K]

X3

»

RE





Figure 3‑6
3.4.4 Record 
On the keyboard, click 【AUX】and【1】 at the same time，you can go to DVR record control interface.

Use the joystick to set channel number and status. Click 【ENTER】 to save current setup. 

3.4.5 PTZ Control and Video Color 

On the keyboard, click 【FN】and【1】at the same time, you can go to the DVR aux interface.
Use the joystick to select PTZ or video color set.

Click 【AUX】and【2】 button at the same time, you can directly go to the PTZ control interface.

3.4.6 Tour
On the keyboard, click 【FN】and【2】at the same time, you can control the tour function of the DVR.

Note

This function is only valid when the tour function of the DVR is enabled. You need to go to the Setting->Display to enable tour function. 

3.4.7 1-Window/Multiple-Window Switch 
· On the keyboard, click a number and 【CAM】 at the same time, you can go to the specified channel. For example, click number 【1】and【CAM】 to go the channel 1.

· On the keyboard, click a number and 【MULT】 at the same time, you can enable multiple-channel display mode. For example, click number 【4】and【MULT】,system goes to 4-window display mode. 

4 Network Keyboard Control Speed Dome
4.1 Cable Connection 

Please connect keyboard A/B line to speed dome A/B line connection is right.
4.2 Keyboard Setup
a) Please make sure keyboard A/B line and speed dome A/B line connection is right.

b) Set speed dome address so that the speed dome address and the keyboard RS485 address are the same. 
c) Go to the zones interface of the keyboard. Please refer to chapter 2.3.2
Note
· Please select protocol according to the speed dome type. The default setup is DH-SD.
· Please login WEB of the speed dome, from main interface->Setup->System->PTZ to view speed dome address, protocol, COM setup and etc. 
d) After the setup, click 【ESC】 to exit control point control interface. System pops up a dialogue box you can click 【ENTER】 to save data or click 【ESC】 to cancel setup. 

4.3 Operation 

On the keyboard, click 【ID】 to go to the control point interface to set speed dome ID to connect to the speed dome.  
After successful connection, the interface is shown as below. See Figure 4‑1.
[image: image32.png]4

A s

=
’ Elrous

zoom





Figure 4‑1
· Step: The move speed. Input number buttons on the keyboard and then click 【ENTER】 to confirm. The speed ranges from 1 to 8. The step 1 is faster than step 8.

· The lens movement supports 8 directions. It includes: up/down/left/right/top left/top right/bottom left/bottom right. Please use the joystick to control. 

4.3.1 Speed Dome Menu 
On the keyboard, click 【OTHER】 button. The interface is shown as below. See Figure 4‑2.
[image: image33.png]@Close Nenu
@Menu Ok
@lfenu Up

D.
®Menu Left
@lfenu Right
®Light On





Figure 4‑2
In this interface, you can use the joystick to control the menu interface of the preview interface of the speed dome WEB.
4.3.2 Iris/Zoom/Focus
4.3.2.1 Iris 
On the keyboard, click 【IRIS】 button, use button 【+】and【-】or turn joystick up/down to control the iris.
4.3.2.2 Focus

On the keyboard click 【FOCUS】 button, use button 【+】and【-】or turn joystick left/right to control the focus.
4.3.2.3 Zoom
On the keyboard click 【ZOOM】 button, use button 【+】and【-】or turn joystick clockwise or counter clockwise  to control the zoom.
4.3.3 Preset
On the keyboard click 【PRESET】, the interface is shown as below. See Figure 4‑3.
[image: image34.png]



Figure 4‑3
4.3.3.1 Set Preset
a) Use joystick to control PTZ to set preset position.
b) On the keyboard click 【PRESET】 button, you can go to the preset interface.

c) Input preset number and then select set preset, click 【ENTER】 to save preset setup. 

4.3.3.2 Go to preset 

Select Preset and then input preset number, click 【ENTER】 to go to the corresponding preset position. 

4.3.3.3 Delete preset 

Input the preset number at the preset No. item and then select Delete, click 【ENTER】 go remove it.

4.3.4 Scan 
a) On the keyboard click 【SCAN】 button, the interface is shown as below. See Figure 4‑4.
[image: image35.png]Scan





Figure 4‑4
b) Select Set and then click 【ENTER】, you can see an interface shown as in Figure 4‑5.
[image: image36.png]Set Right





Figure 4‑5
c) Select left limit and right limit, click 【ENTER】 to set.

d) Use joystick to control PTZ to set limit. 

· Select start and then click 【ENTER】,system begins to scan according to the left/right limit you set.
· Select stop and then click 【ENTER】, system stops scan. 

4.3.5 Pan

On the keyboard click 【PAN】 button, the interface is shown as below. See Figure 4‑6.
[image: image37.png]Fan





Figure 4‑6
· Select start and then click 【ENTER】button, system begins pan operation. 
· Select stop and then click 【ENTER】button, system stops pan operation. 

4.3.6 Tour
4.3.6.1 Set Tour 
a) On the keyboard click 【TOUR】 button, the interface is shown as below. See Figure 4‑7.
[image: image38.png]Start
Stop





Figure 4‑7
b) Select set item, the interface is shown as in Figure 2‑9.
[image: image39.png]Presethdd
PresetDel





Figure 4‑8
c) Input a value at the tour No.
d) Add preset to the tour. 

· Select preset No. and then input a preset number.

· Select preset add and then click 【ENTER】 to confirm. Now the preset is added to the tour. 

4.3.6.2 Tour

Select tour and then input tour number.

· Select start and then click 【ENTER】, system begins tour.

· Select stop and then click 【ENTER】,system stops tour. 

· Select delete and then click 【ENTER】, system removes the tour. 
4.3.7 Pattern

4.3.7.1 Pattern 
a) On the keyboard click 【PATTERN】, the interface is shown as below. See Figure 4‑9.
[image: image40.png]Pattern
N o -
Start
Stop





Figure 4‑9
b) Select tour number and then input a value.

c) Select tour set button and then click 【ENTER】 button. 

d) Use joystick to set pattern and then click 【ENTER】 to end. The speed dome can memorize your whole operation. 
4.3.7.2 Start pattern 

Select tour No. and then input a tour number, select start and then click 【ENTER】 button, system begins pattern operation.
4.3.7.3 Stop pattern

Select tour No. and then input a tour number, select stop and then click 【ENTER】 button, system stops pattern operation. 

5 Keyboard Control NVS

5.1 Connection 

Connect the network cable to the RJ45 network port of the keyboard, now you can implement network control and upgrade. 

5.2 Keyboard Setup  
a) On the keyboard to set control point setup, Please refer to chapter 2.3.2. The link type shall be NET.

Note

· Please input NVS IP address and the default port value is 37777.

· The protocol usually is DH-2. It is the network keyboard and the 2nd control keyboard protocol.  The DVR COM protocol is network keyboard protocol.

b) After the setup, click 【ESC】 to exit.

5.3 Operation
5.3.1 Login 

a) You can use ID, device name, or IP address to login the device. 

b) On the keyboard click 【ENTER】, you can go to the menu option interface.

c) Select control point. 

d) Input ID, device name, or IP address and then click 【ENTER】 to search. 

· System automatically connects to the NVS if the search is successful. Please note you need to input corresponding administrator password of the NVS to login. See Figure 2‑10. 
· You can see the corresponding dialogue box such as the “IP address does not exist. Please input again!” Please check the information and input again. 

Tips
On the keyboard click 【ID】and then input ID, click 【ENTER】 button to quickly find ID and connect to the device you want to control. 

5.3.2 Exit

On the keyboard click 【ID】 and then click 【ESC】, you can exit menu. 

5.3.3 1-Window/Multiple-Window Switch 
· On the keyboard, click a number and 【AUX】 at the same time to select corresponding VGA port number. For example, click number 【1】and【AUX】 to go to video from the VGA 1.

· On the keyboard, click a number and 【CAM】 at the same time, you can go to the specified channel. For example, click number 【1】and【CAM】 to go the channel 1.Or you can click a number and 【SETUP】 to go to a channel. But the value here ranges from 1 to 9. That is to say, you can go to channel 1-channel 9 only. 

· On the keyboard, click a number and 【MULT】 at the same time, you can enable multiple-channel display mode. For example, click number 【4】and【MULT】,system goes to 4-window display mode. 

6 Keyboard Upgrade

6.1 Control Keyboard Upgrade
Control keyboard does not support remote upgrade. Please ship your control keyboard back to our factory to upgrade.
6.2 Network Keyboard Upgrade
Before upgrade, please make sure your network keyboard is well connected. You can use command “PING” to test network connection, the return value TTL should be less than 255.

Please download upgrade file to your local PC. You can download upgrade file from our website or from our local service engineer.

Double click RECUpgrade.exe to begin upgrade. See Figure 6‑1.
[image: image41.png]<L DVR Upgrade Tool Verl. 16

||
[fe 1z 1 Logout

Carrent progress

me [ mios | web [ soot | mork [ tome | oes |





Figure 6‑1
Please input network keyboard IP address and port. Then click open files button to select upgrade file. 
After selection, click open to add current file to the address column.

System automatically recognizes upgrade file, you can click BIOS button to begin upgrade. System pops up upgrade process bar for your reference.
After successful upgrade, click OK to finish operation. 
Tips: 

Use you mouse to click the upgrade file; drag it to the “open files” column and then release. You can see the file path and current file is open.

System pops up alert information if it is not keyboard upgrade file. 
7 Make COM Cable
The keyboard standard accessories bag includes the following items:
· One DB9

· Seven DB9

· One six-pin crystal port

· One 25-pin (DB25)

· One 485 switch box. (Including one power sourcing.) 
7.1 Make RS232 COM Cable  
Please refer to the following figure for RS232 port information. See Figure 7‑1.
[image: image42.png]



RS232 Serial Port                                          RS232 Serial Port 
（DB9）                                                           （DB9）
Figure 7‑1
7.2 Make RS485 COM Cable 
You can refer to Figure 7‑2 if you are connecting the keyboard with the switcher box. 

                                             RS485 Port of Keyboard                      RS485 Switch Box Port
                                                                                                                                                （Green alarm socket）                                  （6-pin crystal ）
[image: image43.png]R+ R- T- T+|A1 B1 A2 B2

R





Figure 7‑2
7.3 25-pin Port and Switch Box Connection 
Connect switch 25-pin box RS232 port with DVR RS232 port. (25-pin outlet supports maximum twelve RS232 ports. Switch is just for sending not for receiving. So you can use two cables: GND and TXD) 
Please refer to the following sheet if you are connecting 485 switch box to DVR See Figure 7‑3.
 RS485 Switch Box 25-Pin Port                                   RS232 Port
                              （DB25）                                                   （DB9）
[image: image44.png]



Figure 7‑3
Note: you can use standard RS232 port to connect keyboard with DVR.
Note:

· This quick start guide is for reference only. Slight difference may be found in the user interface.

· All the designs and software here are subject to change without prior written notice.
· If there is any uncertainty or controversy, please refer to the final explanation of us.
· Please visit our website for more information.

































































































































































































































































































































Short circuit





1 2 3



























































PAGE  
1

